Etapes du commissioning

Retrouver le faisceau 50 MeV, 100 pC, 10 Hz

  - Vérification Canon HF

    - Image analysis :

      - Library or Ds

      - Few difficulties

        - Remove dark current

        - ROI

        - Projections ? 2D fitting …

    - Laser Alignment :

      - BPMs = f(phase)

      - Yag = f(phase)

      - Centered with DCurrent

    - Vérification Alignement solénoïde

      - With BPMs : did not worked (signal change with beam size)

      - Use Yag but need to adapt gun phase to compensate defocusing

    - Mesure laser (taille, énergie)

      - Size : to minitor

      - Energy : Photodiod / PMT

        - Need Calibration /

        - Need Monitoring

    - Courant obscurité

      - Measurement

        - Charge

        - Yag

    - Alignement laser

    - Scan charge phase

      - Good working point

      - Charge measurement ICT : Integration / DC

      - Which script : “The Correct ONE ”

    - Mesure de la taille transverse en fonction du solénoïde sur YAG

    - Mesure Energie en sortie canon

    - Comparaison simulations

  - Vérification Section HF

    - Alignement sortie section (visu YAG)

    - Transport de la charge, mesure de charge

      - Vs : Phase Section

      - Vs : Soleno

    - Quad à bien aligner

    - Transport à travers les quadrupôles bien alignés

    - Optimisation dispersion Energie

      - Vs : Phase Section

      - Vs : Soleno

    - Mesure émittance

      - @ different Soleno B

    - Comparaison simulations

  - Transport ligne de transfert (passage dipôle)

    - Vérification alignement

    - Mesure Energie et dispersion en Energie

    - Transport jusqu’à injection

    - Mesure longueur de paquet ?

    - Mesure paramètres de twiss

    - Matching

    - Correction orbite

- Augmentation de la charge à 500 pC  ++

  - Vérification Canon HF

    - Mesures à plus haute charge

# RING :

here is the list of code and tools used by us most often for the ring.

Injection to ring (start DS): DPI,Septum,  Kicker, Steerer 4 and 5.
Code (Matlab): RCDS optimizer.
Path:  /data/shared/commissioning_scripts/Ring/Injection_Scans/First_Turns/RCDS

MML tools:  main tools are plotorbit, put_quad, tune_plot_update.
Code (Matlab): works with mml
Path: /data/shared/commissioning_scripts/Ring/Injection_Scans/

Libera settings interface:
Code (Pyhton): /data/shared/Libera/labodg ; python libera_config.py

Open TbTSum:
Command: interfaces->ADC BPM

Open TbTX for bpm3 :
Command: terminal: taurusform ri-c1/dg/bpm.03-lib.03/TbTX

Oscilloscope:
Command:  browser 192.168.XX.22

RF Cavity interface:
Command: interfaces-> RF Cavity

Need to optimize 6d Orbit : Manually  correct energy, phase : attenuator, RF phase.
Reduce synchrotron oscillations.